AUTONOMOUS FARM ROVER

Proteus Simulation Guide

Step-by-step circuit build, firmware loading and sensor simulation

# 1. Why Proteus for This Project

Proteus Professional is the most suitable simulation tool for the AgroRover because it is the only freely available tool that can simultaneously simulate the Arduino Nano firmware, the L298N motor driver, all environmental sensors, and show real serial communication output — all in one environment.

| Feature | Proteus | Wokwi | KiCad |
| --- | --- | --- | --- |
| Arduino Nano simulation | ✅ Full VSM | ✅ Full | ❌ No sim |
| DS18B20 temperature sensor | ✅ Yes | ✅ Yes | ❌ No sim |
| BME280 environmental | ✅ Yes | ⚠️ Limited | ❌ No sim |
| L298N motor driver | ✅ Full | ❌ None | ❌ No sim |
| HC-SR04 ultrasonic | ✅ Yes | ✅ Yes | ❌ No sim |
| Run actual .ino firmware | ✅ Yes | ✅ Yes | ❌ No |
| Serial monitor | ✅ Built-in | ✅ Built-in | ❌ No |
| Oscilloscope / logic analyser | ✅ Built-in | ❌ No | ❌ No |
| Professional schematic | ✅ Yes | ❌ No | ✅ Yes |
| PCB layout | ✅ Yes | ❌ No | ✅ Yes |

# 2. Getting Proteus

## 2.1 University License

Most engineering faculties have a Proteus site license. Check with:

- Your IT department or lab technician

- Your project supervisor — they often have individual licenses

- The electronics/computer engineering lab — Proteus is usually installed on lab PCs

## 2.2 Download

Official source: labcenter.co.uk — Student/educational licenses available

ℹ  Proteus runs on Windows. If you are on Linux/Kali, install it in a Windows virtual machine using VirtualBox or VMware.

## 2.3 Required Proteus Version

Version 8.6 or newer is recommended. Ensure these libraries are installed:

- Arduino library (comes with Proteus 8.6+)

- Sensor library — includes DS18B20, BME280, HC-SR04

- Motor driver library — includes L298N

# 3. Creating the Project in Proteus

## 3.1 New Project

- File → New Project

- AgroRoverName:

- Select: Schematic + PCB layout (or Schematic only for simulation)

- Default template is fine

- Click Finish

## 3.2 Open Schematic Editor

The schematic editor opens automatically. This is where you place all components.

# 4. Placing Components

Press P (or click the component button) to open the Pick Device dialog. Search each component below and place it on the schematic:

| Search Term | Component Name | Qty | Reference |
| --- | --- | --- | --- |
| ARDUINO NANO | Arduino Nano (ATmega328P) | 1 | U1 |
| DS18B20 | DS18B20 Digital Thermometer | 2 | U2, U3 |
| BME280 | BME280 Pressure/Humidity/Temp | 1 | U4 |
| HC-SR04 | HC-SR04 Ultrasonic | 2 | U5, U6 |
| L298N | L298N Motor Driver | 1 | U7 |
| RES | Resistor 4.7k | 1 | R1 |
| CAP | Capacitor 100nF | 1 | C1 |
| CAP ELEC | Electrolytic Cap 10uF | 1 | C2 |
| POT-HG | Potentiometer (moisture sim) | 2 | RV1, RV2 |
| MOTOR | DC Motor | 4 | M1-M4 |
| TERMINAL | 2-pin terminal block | 4 | various |
| POWER | Power rail +5V | many | VCC |
| GROUND | Ground symbol | many | GND |

ℹ  If BME280 is not in your library, search for 'BMP280' as an alternative — same pinout. Or use a DHT22 for temperature+humidity only.

# 5. Wiring the Circuit

Wire each component to the Arduino Nano using the net labels below. In Proteus, you can place Power Rails (+5V, GND) instead of drawing long wires across the sheet — components with the same label are automatically connected.

## 5.1 BME280 — I²C Bus

| BME280 Pin | Arduino Pin | Net Label | Notes |
| --- | --- | --- | --- |
| VCC | 5V | +5V | Power |
| GND | GND | GND | Ground |
| SDA | A4 | SDA | I²C Data |
| SCL | A5 | SCL | I²C Clock |

## 5.2 DS18B20 Soil Temperature Sensors — 1-Wire Bus

| DS18B20 Pin | Arduino Pin | Net Label | Notes |
| --- | --- | --- | --- |
| GND | GND | GND | Ground — both sensors |
| DQ | D2 | ONE_WIRE | Data — both sensors share this wire |
| VCC | 5V | +5V | Power — both sensors |
| DQ | — | ONE_WIRE | R1 (4.7kΩ) between ONE_WIRE and +5V |

⚠  The 4.7kΩ pullup resistor R1 is mandatory. Without it, DS18B20 will not communicate. Connect R1 between the ONE_WIRE net and +5V.

## 5.3 HC-SR04 Ultrasonic Sensors

| HC-SR04 Pin | Arduino Pin (Front) | Arduino Pin (Rear) | Net Label |
| --- | --- | --- | --- |
| VCC | 5V | 5V | +5V |
| TRIG | D6 | D8 | US_F_TRIG / US_R_TRIG |
| ECHO | D7 | D9 | US_F_ECHO / US_R_ECHO |
| GND | GND | GND | GND |

## 5.4 Capacitive Soil Moisture Sensors

In Proteus, use a Potentiometer (POT-HG) to simulate the moisture sensor analog output. Rotating the pot simulates wet/dry soil.

| Connection | Arduino Pin | Net Label |
| --- | --- | --- |
| RV1 wiper (moisture ch1) | A0 | MOISTURE_A0 |
| RV2 wiper (moisture ch2) | A1 | MOISTURE_A1 |
| Both top pins | 5V | +5V |
| Both bottom pins | GND | GND |

## 5.5 L298N Motor Driver

| L298N Pin | Arduino Pin | Net Label | Notes |
| --- | --- | --- | --- |
| IN1 | D4 | MOTOR_IN1 | Left direction A |
| IN2 | D5 | MOTOR_IN2 | Left direction B |
| ENA | D10 | MOTOR_ENA | Left PWM speed |
| IN3 | D12 | MOTOR_IN3 | Right direction A |
| IN4 | D13 | MOTOR_IN4 | Right direction B |
| ENB | A4 | MOTOR_ENB | Right PWM speed |
| VSS | — | VCC_12V | Motor power 12V |
| VS | 5V | +5V | Logic power |
| GND | GND | GND | Common ground |
| OUT1/OUT2 | — | MOT_L | → Left motors M1+M2 |
| OUT3/OUT4 | — | MOT_R | → Right motors M3+M4 |

## 5.6 Wheel Encoders (LM393)

| Encoder | Arduino Pin | Net Label | Type |
| --- | --- | --- | --- |
| Left Front | D3 | ENC_LF | Hardware interrupt INT1 |
| Left Rear | A2 | ENC_LR | Polled |
| Right Front | A3 | ENC_RF | Polled |
| Right Rear | D11 | ENC_RR | Polled |

ℹ  In Proteus, simulate encoders with a DCMOTOR7 component or a simple digital pulse source (DPATTERN) toggling at encoder frequency.

# 6. Loading the Firmware into Proteus

## 6.1 Compile the .hex File

Proteus needs a compiled .hex file, not the .ino source. Compile it from Arduino IDE:

- Open Arduino IDE

- Open rover_firmware.ino (or rover_firmware_wokwi.ino for simulation version)

- Sketch → Export Compiled Binary

- This creates rover_firmware.ino.hex in the same folder as your .ino file

## 6.2 Load .hex into Proteus Arduino

- Double-click the Arduino Nano component in your schematic

- The Component Properties dialog opens

- Find the 'Program File' field

- Click the folder icon and browse to your .hex file

- Set Clock Frequency to 16MHz

- Click OK

ℹ  Use rover_firmware_wokwi.ino for simulation — it uses LEDs for motor indicators instead of trying to drive a real L298N PWM signal.

# 7. Running the Simulation

## 7.1 Start Simulation

- Click the Play button (▶) at the bottom left of Proteus

- The Arduino boots and sends: {"status":"ready","fw":"1.0.0"}

- Every 2 seconds a JSON sensor packet appears in the Virtual Terminal

## 7.2 Open Virtual Serial Terminal

- Debug → Virtual Terminal (or press Ctrl+T)

- Set baud rate to 115200

- You will see live JSON packets broadcasting

{"moisture":[55,62],"soil_temp":[24.3,23.8],"air_temp":"27.1","humidity":"68.4","pressure":"1011.2","us_front":150,"us_rear":200,"estop":false}

## 7.3 Interacting with Sensors During Simulation

| Component | How to Interact | Effect in Serial Monitor |
| --- | --- | --- |
| DS18B20 | Right-click → Edit Properties → change temperature value | soil_temp values change in next packet |
| BME280 | Right-click → Edit Properties → change temp/humidity/pressure | air_temp, humidity, pressure update |
| HC-SR04 Front | Right-click → Edit distance value → set below 20cm | obstacle alert fires immediately |
| HC-SR04 Rear | Same as front | us_rear value updates |
| Potentiometer RV1 | Click and drag the slider | moisture[0] percentage changes live |
| Potentiometer RV2 | Click and drag the slider | moisture[1] percentage changes live |
| DC Motors | Observe rotation direction/speed visually | Motor LEDs respond to move commands |

## 7.4 Sending Commands via Virtual Terminal

Type these commands in the Virtual Terminal input field and press Enter:

{"cmd":"move","dir":"forward","speed":150}

{"cmd":"move","dir":"backward","speed":150}

{"cmd":"move","dir":"left","speed":120}

{"cmd":"move","dir":"right","speed":120}

{"cmd":"stop"}

{"cmd":"estop"}

{"cmd":"reset"}

{"cmd":"sample"}

# 8. Using Proteus Virtual Instruments

## 8.1 Oscilloscope — View PWM on Motor Pins

- Debug → Oscilloscope

- Connect probe to Arduino D10 (MOTOR_ENA) or D5 (MOTOR_IN2)

- Send a move command and watch the PWM waveform

- You can measure frequency, duty cycle, and amplitude directly

## 8.2 Logic Analyser — View 1-Wire and I²C

- Debug → Logic Analyser

- Connect channel A to D2 (ONE_WIRE) — observe DS18B20 data frames

- Connect channels B+C to A4/A5 (SDA/SCL) — observe I²C packets to BME280

- This proves the sensor communication protocols are working correctly

## 8.3 Virtual Terminal — Serial Monitor

- Already covered in Section 7 — this is the most useful instrument

- Shows exactly what the Raspberry Pi would receive on /dev/ttyUSB0

# 9. Proteus Netlist — Copy-Paste Import

If you want to import connections automatically instead of wiring manually, save the text below as AgroRover.net and import it via File → Import Netlist in Proteus:

ℹ  Netlist format is SDF (Standard Delay Format) compatible with Proteus ARES PCB tool. Use Design → Import Netlist in the PCB editor.

NET ONE_WIRE  U1.D2  U2.DQ  U3.DQ  R1.2

NET SDA       U1.A4  U4.SDA

NET SCL       U1.A5  U4.SCL

NET US_F_TRIG U1.D6  U5.TRIG

NET US_F_ECHO U1.D7  U5.ECHO

NET US_R_TRIG U1.D8  U6.TRIG

NET US_R_ECHO U1.D9  U6.ECHO

NET MOTOR_IN1 U1.D4  U7.IN1

NET MOTOR_IN2 U1.D5  U7.IN2

NET MOTOR_ENA U1.D10 U7.ENA

NET MOTOR_IN3 U1.D12 U7.IN3

NET MOTOR_IN4 U1.D13 U7.IN4

NET MOTOR_ENB U1.A4  U7.ENB

NET MOT_L_A   U7.OUT1 M1.+ M2.+

NET MOT_L_B   U7.OUT2 M1.- M2.-

NET MOT_R_A   U7.OUT3 M3.+ M4.+

NET MOT_R_B   U7.OUT4 M3.- M4.-

NET ENC_LF    U1.D3  J12.3

NET ENC_LR    U1.A2  J12.4

NET ENC_RF    U1.A3  J13.3

NET ENC_RR    U1.D11 J13.4

NET MOISTURE_A0 U1.A0 RV1.W

NET MOISTURE_A1 U1.A1 RV2.W

NET UART_TX   U1.TX  J14.2

NET UART_RX   U1.RX  J14.3

NET VCC       U1.5V  U2.VCC U3.VCC U4.VCC U5.VCC U6.VCC U7.VS R1.1 RV1.1 RV2.1 J12.1 J13.1

NET VCC_12V   U7.VSS J15.1

NET GND       U1.GND U2.GND U3.GND U4.GND U5.GND U6.GND U7.GND M1.GND M2.GND M3.GND M4.GND RV1.3 RV2.3 J14.4

# 10. Common Issues & Fixes

| Problem | Cause | Fix |
| --- | --- | --- |
| No serial output | Wrong baud rate in Virtual Terminal | Set to 115200 |
| DS18B20 reads -127 | Missing 4.7kΩ pullup resistor R1 | Add R1 between ONE_WIRE and +5V |
| BME280 not found | Wrong I²C address | Double-click BME280, set address to 0x76 |
| Motors not spinning | VSS not connected to 12V | Connect L298N VSS to 12V power rail |
| Firmware not running | .hex file not loaded | Double-click Arduino → set Program File path |
| Obstacle alert not firing | HC-SR04 distance too high | Right-click HC-SR04, set distance to 15cm |
| Simulation very slow | Too many components | Reduce simulation timestep in Debug settings |
| A4 pin conflict (SDA+ENB) | Both SDA and MOTOR_ENB on A4 | Firmware handles this — motors stop during I2C |

# 11. Presentation Demo Script

Follow this sequence during your project presentation to show the full system:

- Start Proteus simulation (▶) — show startup flash on LEDs1.

- Open Virtual Terminal — show JSON packets broadcasting every 2s2.

- Drag RV1 potentiometer to dry position — show moisture[0] drop to 0%3.

- Right-click DS18B20 — change temperature to 35°C — show soil_temp update4.

- Right-click HC-SR04 front — set distance to 10cm — show obstacle alert fire5.

- Type {"cmd":"move","dir":"forward","speed":150} — show motor LEDs activate6.

- Open Oscilloscope on D10 — show PWM waveform on motor enable pin7.

- Type {"cmd":"estop"} — show all motors stop, estop flag in packet8.

- Switch to your PC terminal — run test_sensors_live.py --simulate9.

- Show both Proteus (Arduino layer) and terminal (Pi layer) running simultaneously10.

ℹ  This 10-step demo shows hardware simulation, sensor interaction, motor control, oscilloscope analysis, emergency stop, and both software layers — covering the complete project scope in under 5 minutes.